ABBA-MANIA (7)  TECHNICAL RIDER

All items included within this rider are needed in order to maximize the event production. Alternate equipment is flexible only if approved in advance by management sound technician prior to arrival. Please contact Garry Lichach at (905) 334-1333.

ABBA-Mania is made up of the 4 lead Abba characters backed by guitar, bass, keys and drums.

STAGE RISERS

2  4’ x 8’ decks at 32” high with carpet (Drums)

4  4’ x 8’ decks at 24” high (Guitar & Bass)

4  4’ x 8’ decks at 12” high (Back Vocals & Orchestra)

Priority riser is Drums.

BACKLINE

1  5 piece drum kit – Hi hats – Ride – Crash # 1 – Crash # 2 – Hardware – Tune key

1 RD 700 or 800 Electric keyboard  - Stand – Sustain pedal

2  2 ch. Fender Twin Guitar amp 2000 series with foot switch

1  small combo bass amp

2 Fender Stratocoster guitars

1  five string Fender bass guitar

3 guitar stands

Sound and Lights, including 8 monitors, and including 2 WIRELESS HAND HELD MICS AND STANDS.

FOH AUDIO	     ( Monitor Control at FOH )

One	32 ch.  Console, ( 4 x Monitor Mixes )

One	FOH FX rack, EQ’s – Multi FX – Gates – Limiter

Four	EQ’s for monitor mixes

One	Lot patch cables for hookup

Two	Mid hi speaker cabinets per side with correct power amplifiers. (Minimum)

Two	Sub speaker cabinets per side with correct power amplifiers. (Minimum)

One	32 ch. snake. Distance from stage to FOH mix position.

Two	AC circuits connected with power bars

MONITORS	     (Monitor Control at FOH)

Eight Monitors.

One lot of cable for hook up ( 32’ x 16’ stage size )

Correct amplifier match up to 4 mixes and consider adjustments if drum monitor is full range.

STAGE HOOKUP   (Items in this category must refer to stage plot)

6	Direct boxes

2	Straight vocal mic stands (Down stage girls Abba Mania)

1	Mic – Stand – cables packaged for stage hookup

2	UHF wireless handheld mics and clips

Remaining items as per stage plot equipment flexible to make event work.

LIGHTING  (Minimum 48 K lighting package)

4	Automated fixtures (Mirror or moving heads OK)

2	Follow spot (Minimum 1K)

1	Hazer type fogger

1	Console for conventional lighting minimum 8 Sub Groups)

1	Console for automated lighting (Same console for conventional OK)

All cable for hookup including 220 connections  (Flexibility OK)

COMMUNICATIONS   (1 ch. base, headsets and packs for the following positions)

1	FOH Audio

1	FOH Lighting

2	Spotlights

1	Stage

All cables for hookup (XLR & AC)